Reliability Compare and Contrast Summary

The Reliability Compare and Contrast document was developed to assist the NCTPC PWG in determining if there were significant differences in the reliability assumptions used by PEC and Duke in their transmission planning processes that may need to be addressed to produce the 2006 NCTPC reliability transmission plan. The comparison was performed by categories. A summary of each follows.

Calendar

The annual planning process calendar for each company is similar. The annual process for PEC divides screening into two sets of studies 1) near-term (1-5 yrs) during the 1st quarter and 2) long-term (6-10 yrs) in the 3rd quarter. Duke’s process does not divide the screening process. Since this difference would impact the ability to begin the 2011 reliability study under the NCPTC, Progress will adjust its planning calendar.

Case Development

Differences exist in the assumptions for roll-over and future DNRs (designated network resources). Duke includes LSE's DNR projections in the base cases. PEC assumes roll-over of existing DNRs but does not include changes to existing DNRs or new DNRs in base cases until the OATT requirements are completed, initiated by OASIS queuing. The PWG plans to assess the impact this will have on the 2011 reliability study as the study proceeds. The PWG will follow-up on any recommended changes believed necessary.

Duke’s base cases assume a dispatch priority of LSE resources provided by the LSEs. PEC’s base cases include all LSE imports flow.  The PWG plans to assess the impact this will have on the 2011 reliability study planned for completion in 2006. The PWG has asked LSE’s for such a priority list in preparation of the 2011 study case. PWG will follow-up on any changes believed necessary but no changes are recommended at this time.

Both Duke and PEC use future dummy generation when additional system resources are needed to serve control area load when plans for future resources may not be complete. Duke locates dummy generation at buses based on generator interconnection queue. PEC locates their generation at a 500 kV bus to reduce the impact it may have on the system.  Since the 2011 case does not have future generation needs the PWG defers making any recommendations to this assumption.

Assessment Practices

Duke utilizes 12-hour and emergency ratings under some contingencies. PEC uses a continuous rating for all analysis. No changes are currently planned to address this difference.

Maximum import assumptions are different for PEC and Duke. Duke includes assessment with added imports in the amount of outaged generation. PEC assesses with base case imports plus the simultaneous import of all TRM obligations. No changes are currently planned to address this difference.

Due to the impact on phase angle from significant parallel path flow PEC monitors the Richmond-Newport 500 kV Line phase angle. Duke does not currently monitor phase angle but is evaluating the criteria. No changes are currently planned to address this difference.

CBM/TRM

CBM is zero for both PEC and Duke. Both Duke and PEC reserve VACAR Reserve Sharing on respective interfaces.  In addition PEC includes a parallel path component. Also for non-VACAR interfaces PEC utilizes s system inrush response on that interface.  PEC also assumes simultaneous import of all TRM in addition to base imports.   No changes are currently planned to address this difference.

Ratings

There are some minor differences in the assumptions made in determining the continuous ratings of lines and transformers.  No changes are currently planned to address differences.

Conclusion

The comparison of PEC’s and Duke’s reliability planning practices includes many similarities but there are some significant differences. The consequences associated with resolving some of these differences are complex. The PWG recommends no other changes than the change mentioned above that PEC will make in its planning calendar. The PWG may recommend other changes as the joint studies proceed.