Project Name:	Franklinton 115 kV Substation, Install  18 MVar 115 kV  Capacitor

Project Description

Install 18 MVar 115 kV capacitor at Franklinton 115 kV Substation

| Projected In-Service Date for Base Reliability Plan | June 2011 |
| --- | --- |
| Earliest Projected In-Service Date For Resource Supply Option | June 2011 Not impacted by import |
| Project Lead Time | 2 years |
| Estimated Cost | $0.5 M |

Narrative Description of the Need for this Project

By 2011 summer, a loss of the Falls - Franklinton 115 kV line will result in unacceptable voltage in the Wake Forest area. Installing a 115 kV capacitor at the Franklinton 115 kV Substation will correct contingency voltage in the area. This issue is not significantly impacted by Resource Supply options.

Transmission Solutions Considered

115 kV – 18 MVar capacitor in the Wake Forest area.

Why this Project was Selected as the Preferred Solution

Installation of a 115 kV capacitor at Franklinton 115 kV resolves contingency voltage.